INTERNAL REFERENCE DOCUMENT

Emergent Room Resonance Analysis

Code-Level Reference: What Happens, Where, and Why

Kill The Ring  •  v0.159.0  •  March 20, 2026

Author: Don Wells

CONFIDENTIAL — NOT FOR DISTRIBUTION

Table of Contents

1. Quick Reference Card

2. Discovery Summary

3. Code Path: How Room Modes Become Advisories

4. Algorithm Score Mapping (Source-Level)

5. Gate Bypass Analysis (Source-Level)

6. Configuration Values That Matter

7. How to Reproduce the Behavior

8. Productization Roadmap

9. File Index

10. Related Documents

1. Quick Reference Card

⚠ TL;DR KTR at high sensitivity (threshold ≤ 8 dB) with no feedback loop detects room resonance modes as feedback. All 6 algorithms unanimously score room modes identically to feedback. Room mode gates are disabled by default (roomPreset = 'none'). The system generates PEQ advisories that constitute a room correction EQ profile.

| Discovery date | 2026-03-20 |
| --- | --- |
| Trigger condition | ringOut mode (2 dB threshold), no mic→speaker loop |
| Root cause | Room resonances ≡ feedback at spectral level |
| Why gates don't fire | roomPreset = 'none' (default) disables room physics |
| Output | PEQ advisories: frequency, Q, cut depth = room correction EQ |
| Key files | feedbackDetector.ts, algorithmFusion.ts, classifier.ts, constants.ts |
| Formal theorem | ‖S_fb − S_rm‖₂ → 0 for persistent narrowband peaks |
| Patent status | Provisional filed 2026-03-20 (Docket WELLS-2026-001) |
| AES paper | Drafted (2026-03-20-emergent-room-resonance-analysis.docx) |

2. Discovery Summary

While testing KTR in Ring Out mode (2 dB prominence threshold) with no active feedback loop — the microphone was capturing ambient room sound only, with no routing to speakers — the system generated sustained EQ advisories for persistent spectral peaks.

Investigation revealed these peaks correspond to room acoustic resonance modes (standing waves between parallel room surfaces). All six detection algorithms unanimously classified the room modes as feedback because room resonances and acoustic feedback produce physically identical spectral signatures:

• Both are persistent (stable over many frames)

• Both are narrowband (high Q factor, concentrated at one frequency)

• Both have stable magnitude (low MSD ≈ 0)

• Both have stable phase (high phase coherence ≈ 0.85–0.98)

• Both have clean spectra (low IHR, no interharmonic energy)

• Both are sharp peaks (high PTMR relative to spectral neighborhood)

The only difference is the sustaining mechanism — electroacoustic loop (feedback) vs. wall reflections (room mode) — which is invisible to spectral analysis at the microphone.

3. Code Path: How Room Modes Become Advisories

This section traces the exact code path a room resonance takes through the KTR pipeline, from microphone input to advisory card.

3.1 Audio Capture → Peak Detection (Main Thread)

File: lib/dsp/feedbackDetector.ts (~1,823 LOC)

Function: FeedbackDetector.analyze() — called at 50 fps (every 20 ms)

1. getFloatFrequencyData() → raw spectrum (Float32Array, 8192 bins) 2. applyCompensation() → A-weighting + mic calibration 3. updateNoiseFloor() → EWMA noise floor tracking (decay 0.98) 4. autoGainControl() → adjust gain (attack 0.3s, release 1.0s) 5. For each bin above threshold: prominence = bin_magnitude - prefixSumNeighborhoodAvg(bin) if prominence > settings.feedbackThreshold → candidate peak 6. persistenceScoring() → track across frames speech: 7 frames, music: 13 frames, compressed: 50 frames 7. Room mode passes: it IS persistent, it IS prominent at low threshold 8. postMessage(peak, spectrum, timeDomain) → Worker [transferable]

Key threshold: settings.feedbackThreshold — set by mode preset. Ring Out mode = 2 dB. At this level, typical room modes (6–15 dB prominence) easily exceed the threshold.

3.2 Algorithm Scoring (Worker Thread)

File: lib/dsp/workerFft.ts (~389 LOC) — AlgorithmEngine.computeScores()

File: lib/dsp/algorithmFusion.ts (~919 LOC) — fusion + gates

The worker receives the peak and runs all six algorithms. For a room mode:

AlgorithmEngine.computeScores(peak): msd → ~0.02 (very stable magnitude) ✓ feedback-like phase → ~0.91 (very stable phase) ✓ feedback-like spectral → ~0.03 (very tonal, peaked) ✓ feedback-like comb → varies (may match axial harmonics) ✓ feedback-like ihr → ~0.05 (clean, no interharmonics) ✓ feedback-like ptmr → ~18 dB (sharp isolated peak) ✓ feedback-like fuseAlgorithmResults(scores, contentType): P(feedback) = Σ(score_i × weight_i) / Σ(weight_i) → typically 0.65–0.85 for room modes → above reporting threshold for ringOut mode

3.3 Gate Processing

File: lib/dsp/algorithmFusion.ts — post-fusion gates

File: lib/dsp/classifier.ts (~850 LOC) — classifier gates

Post-fusion gates — NONE FIRE for room modes: IHR Gate (algorithmFusion.ts ~line 485): Condition: harmonicsFound >= 3 AND ihr > 0.35 Room mode: isolated peak, harmonicsFound = 0–1, ihr ≈ 0.05 → NOT ACTIVATED PTMR Gate (algorithmFusion.ts ~line 495): Condition: ptmr.feedbackScore < 0.2 Room mode: sharp peak → feedbackScore ≈ 0.7–0.9 → NOT ACTIVATED CombStability Gate (algorithmFusion.ts ~line 510): Condition: spacingCV > 0.05 over 16 frames Room mode: stationary → CV ≈ 0 → NOT ACTIVATED Formant Gate (classifier.ts ~line 380): Condition: 2+ peaks in F1/F2/F3 bands AND Q ∈ [3,20] Room mode: single isolated peak, not in formant pattern → NOT ACTIVATED Chromatic Gate (classifier.ts ~line 420): Condition: ±5 cents from 12-TET AND coherence > 0.80 Room mode: frequency = c/(2L) etc., NOT on semitone grid → NOT ACTIVATED

3.4 Room Mode Gates — DISABLED BY DEFAULT

File: lib/dsp/classifier.ts — room mode suppression

File: lib/dsp/constants.ts (~1,013 LOC) — default settings

⚠ Critical Configuration Room mode gates (roomModeProximity, modalDensityPenalty, RT60 Q-factor) require roomPreset ≠ 'none'. The DEFAULT_SETTINGS in constants.ts sets roomPreset = 'none'. This means room physics computation is completely bypassed.

// constants.ts — DEFAULT_SETTINGS roomPreset: 'none', // ← This disables all room mode gates roomLength: 0, roomWidth: 0, roomHeight: 0, // classifier.ts — room mode check if (settings.roomPreset !== 'none') { // Compute expected room modes from dimensions // Apply roomModeProximity suppression // Apply modalDensityPenalty // Apply RT60-based Q adjustment } // When roomPreset === 'none': ALL of this is skipped // Room modes pass through unimpeded

3.5 EQ Advisory Generation

File: lib/dsp/eqAdvisor.ts (~402 LOC)

File: lib/dsp/advisoryManager.ts (~292 LOC)

generateEQAdvisory(track): frequency → room mode frequency (Hz) Q → matched to resonance width via bandwidth analysis depth → MINDS algorithm (growth-rate-based) → scaled by ERB: ×0.7 (<500Hz), ×1.0 (500-2kHz), ×1.2 (>2kHz) AdvisoryManager.createOrUpdate(advisory): 3-layer dedup (band, frequency, cents-based proximity) 500ms rate limiting 100-cent proximity grouping → postMessage(advisory) back to main thread → useAdvisoryMap → React render → advisory card displayed // The advisory card IS a room correction filter specification: // "Cut 4.2 dB at 247 Hz, Q = 8.3" // This is exactly what Smaart/REW would recommend.

4. Algorithm Score Mapping (Source-Level)

Exact source locations for each algorithm's computation:

| Algorithm | Primary File | Entry Function | Support File | Implementation |
| --- | --- | --- | --- | --- |
| MSD | workerFft.ts | AlgorithmEngine.computeMSD() | msdPool.ts | Second-derivative stencil, sparse 256-slot pool, LRU eviction |
| Phase Coherence | workerFft.ts | AlgorithmEngine.computePhase() | phaseCoherence.ts | Circular statistics, mean phasor magnitude |
| Spectral Flatness | workerFft.ts | AlgorithmEngine.computeSpectral() | compressionDetection.ts | Geometric/arithmetic mean + kurtosis in ±5 bins |
| Comb Pattern | algorithmFusion.ts | detectCombPattern() | — | Harmonic matching f,2f,3f ±5% tolerance, ≥3 harmonics |
| IHR | algorithmFusion.ts | computeIHR() | — | Sideband energy ratio ±5-15 and ±20-40 bins |
| PTMR | algorithmFusion.ts | computePTMR() | — | Peak dB − median(40-bin window excl. ±2 bins) |
| ML Meta-Model | mlInference.ts | predictCached() | models/manifest.json | MLP 11→32→16→1 929 params, 4KB ONNX |

5. Gate Bypass Analysis (Source-Level)

For each gate, this section details exactly why room resonances bypass it, with source file references.

| Gate | Location | Activation Condition | Factor | Why Room Modes Bypass |
| --- | --- | --- | --- | --- |
| IHR Gate | algorithmFusion.ts ~line 485 | harmonicsFound >= 3 AND ihr > 0.35 | × 0.65 | Room modes are isolated peaks — harmonicsFound = 0 or 1. IHR ≈ 0.05 (clean spectrum). |
| PTMR Gate | algorithmFusion.ts ~line 495 | ptmr.feedbackScore < 0.2 | × 0.80 | Room modes are sharp isolated peaks — PTMR feedbackScore ≈ 0.7–0.9. Gate checks for BROAD peaks, not sharp ones. |
| Comb Stability | algorithmFusion.ts ~line 510 | spacingCV > 0.05 over 16 frames | × 0.25 | Room modes are stationary — spacing never varies. CV ≈ 0. |
| Formant Gate | classifier.ts ~line 380 | 2+ peaks in F1/F2/F3 bands AND Q ∈ [3,20] | × 0.65 | Room modes don't cluster in vocal formant bands. Isolated peaks fail the '2+ bands' requirement. |
| Chromatic Gate | classifier.ts ~line 420 | ±5 cents from 12-TET AND coherence > 0.80 | × 0.60 | Room mode freqs are determined by room dimensions (c/2L), NOT musical scale. Random relation to semitones. |
| Room Mode Gates | classifier.ts ~line 450+ | roomPreset ≠ 'none' | varies | DEFAULT: roomPreset = 'none' (constants.ts). ENTIRE block skipped. No room physics computed. |

6. Configuration Values That Matter

6.1 Mode Presets (constants.ts)

The behavior depends entirely on which mode preset is active. Key values:

| Mode | Threshold (dB) | Silence (dBFS) | MSD Weight | Room Mode Detection |
| --- | --- | --- | --- | --- |
| ringOut | 2 | -70 | 0.33 | Maximum — detects everything |
| monitors | 15 | -45 | 0.33 | High — may detect strong room modes |
| broadcast | 22 | -70 | 0.33 | Medium — unlikely to see room modes |
| speech | 27 | -65 | 0.33 | Low — room modes below threshold |
| worship | 35 | -58 | 0.33 | Very low — room modes invisible |
| liveMusic | 42 | -45 | 0.08 | Lowest — only obvious feedback |

6.2 Critical Default Values

// constants.ts — values that enable the emergent behavior // Room physics (disabled by default) roomPreset: 'none' // Disables room mode gates roomLength: 0 // No room dimensions roomWidth: 0 roomHeight: 0 // Detection thresholds (ringOut mode) feedbackThreshold: 2 // 2 dB prominence — catches everything silenceThreshold: -70 // -70 dBFS — only true silence is ignored // Persistence (content-adaptive) PERSISTENCE_SPEECH: 7 // 140ms — room modes easily persist PERSISTENCE_MUSIC: 13 // 260ms — room modes easily persist PERSISTENCE_COMPRESSED: 50 // 1000ms — room modes still persist // Gate thresholds IHR_GATE_THRESHOLD: 0.35 // Room modes: IHR ≈ 0.05 (below) PTMR_GATE_THRESHOLD: 0.2 // Room modes: PTMR score ≈ 0.8 (above) COMB_CV_THRESHOLD: 0.05 // Room modes: CV ≈ 0 (below)

7. How to Reproduce the Behavior

Step-by-Step Reproduction

1. Open KTR (killthering.com or localhost:3000)

2. Ensure the microphone is NOT routed to any speaker in the same space

3. Click "Ring Out Room" or set mode to ringOut (threshold = 2 dB)

4. Wait 5–15 seconds for ambient sound to excite room modes

5. Observe: advisory cards appear for persistent spectral peaks

6. These are room resonance modes, not feedback

Verification

To verify the detected frequencies are room modes:

• Calculate expected modes: For a 10m × 6m × 3m room, the first axial modes are: 17.2 Hz (L), 28.6 Hz (W), 57.2 Hz (H), and their harmonics. Compare detected frequencies against these predictions.

• Rayleigh equation: f = (343/2) × √((n/L)² + (m/W)² + (p/H)²)

• Toggle room preset: Set roomPreset to 'small' or 'medium' with correct dimensions. Room mode gates will activate and suppress some detections — confirming they were room modes.

8. Productization Roadmap

Potential path to formalizing room analysis as a product feature:

Phase 1: Labeling (Low Effort)

• Add a 'Room Analysis' mode that sets threshold to 2–4 dB and labels advisories as 'Room Resonance' instead of 'Feedback'

• No algorithmic changes — purely UI/labeling

Files to modify: constants.ts (new mode preset), IssuesList.tsx (card labeling)

Phase 2: Room Profile (Medium Effort)

• Auto-detect room dimensions from pattern of detected modes

• Estimate Schroeder frequency from mode density vs. frequency

• Export room correction EQ profile (JSON, CSV, or console-specific format)

Files to modify: acousticUtils.ts (Schroeder estimation), eqAdvisor.ts (room correction export)

Phase 3: Validation (High Effort)

• Side-by-side comparison with Smaart in 5+ venue types

• Quantify accuracy: frequency match (±Hz), Q match, depth match

• Publish results as addendum to AES paper

Phase 4: ML Discrimination (Research)

• Train neural network to distinguish room modes from feedback using contextual features (user-reported loop presence, multi-mic correlation, temporal patterns)

Files to modify: mlInference.ts, scripts/ml/train_fp_filter.py

9. File Index

All source files relevant to the emergent room analysis behavior:

| File | LOC | Role in Room Analysis Behavior |
| --- | --- | --- |
| lib/dsp/feedbackDetector.ts | ~1,823 | Core detection loop, peak detection, prominence, persistence scoring, auto-gain, A-weighting |
| lib/dsp/algorithmFusion.ts | ~919 | 6-algorithm fusion, IHR/PTMR/Comb gates, content-adaptive weights, MINDS depth |
| lib/dsp/classifier.ts | ~850 | 11-feature Bayesian classifier, formant gate, chromatic gate, room mode gates |
| lib/dsp/constants.ts | ~1,013 | ALL tuning constants, 8 mode presets, weight profiles, gate thresholds, DEFAULT_SETTINGS |
| lib/dsp/workerFft.ts | ~389 | AlgorithmEngine — MSD/phase/spectral computation, FFT, phase extraction |
| lib/dsp/dspWorker.ts | ~458 | Worker orchestrator — message handling, temporal smoothing, ML score extraction |
| lib/dsp/eqAdvisor.ts | ~402 | EQ recommendation generation — GEQ/PEQ/shelf, ERB scaling, MINDS algorithm |
| lib/dsp/advisoryManager.ts | ~292 | 3-layer dedup, band cooldown, proximity grouping, rate limiting |
| lib/dsp/msdPool.ts | ~267 | Sparse MSD pool — 256 slots × 64 frames, LRU eviction |
| lib/dsp/phaseCoherence.ts | ~129 | Phase coherence via circular statistics — mean phasor computation |
| lib/dsp/acousticUtils.ts | ~861 | Room mode physics — Rayleigh, Schroeder, RT60, modal density |
| lib/dsp/mlInference.ts | ~180 | ONNX ML inference — lazy loading, predictCached(), 929-param MLP |
| lib/dsp/trackManager.ts | ~466 | Track lifecycle — cents-based association, 100-cent tolerance |
| types/advisory.ts | ~384 | All DSP interfaces — Advisory, DetectorSettings, Track, AlgorithmScores |

10. Related Documents

AES Convention Paper

docs/papers/2026-03-20-emergent-room-resonance-analysis.docx

Academic paper — formal analysis with 17 equations, 20 references, 8 sections. Targeted at AES peer review.

US Provisional Patent

docs/papers/2026-03-20-provisional-patent-ktr-room-analysis.docx

USPTO provisional application — 16 claims (independent: 6-algo fusion; dependent: room analysis). Establishes priority date 2026-03-20. Docket WELLS-2026-001.

Technical Whitepaper

docs/papers/2026-03-20-ktr-room-analysis-whitepaper.docx

Practitioner-focused document — explains the technology and discovery in accessible language with practical use cases and comparison tables.

This Document

docs/papers/2026-03-20-ktr-internal-reference.docx

Internal reference — code-level tracing, exact file/function/line references, reproduction steps, configuration values.

CLAUDE.md

CLAUDE.md

Project intelligence file — architecture overview, coding conventions, version history. Contains high-level overview of the 6-algorithm pipeline.

⚠ Document Maintenance This reference document should be updated whenever algorithm weights, gate thresholds, or the room mode gate logic changes. Key files to watch: constants.ts, algorithmFusion.ts, classifier.ts.