# PA2 Companion Module — Architecture

## System Overview

┌─────────────────────────┐

donewellaudio.com ─────>│                         │

(HTTPS browser)   HTTP  │   Bitfocus Companion    │

│   + PA2 Module          │

Stream Deck XL ────────>│                         │

(buttons/presets)       │   src/main.js           │

│   ├─ TCP :19272 control │──── PA2 Control Port

│   ├─ TCP :19274 meters  │──── PA2 DSP Port

│   └─ HTTP bridge        │

└─────────────────────────┘

The module maintains two simultaneous TCP connections to the PA2:

- Port 19272 — HiQnet text protocol for reading/writing all parameters

- Port 19274 — DSP command interface for real-time meter data (RTA, levels, GR)

## File Map

| File | Lines | Purpose |
| --- | --- | --- |
| src/main.js | ~1200 | InstanceBase subclass: TCP connections, state machine, HTTP bridge, DSP meters |
| src/pa2-protocol.js | ~620 | Protocol layer: response parser, command builder, GEQ band constants |
| src/actions.js | ~850 | All action definitions: mutes, GEQ, PEQ, AFS, compressor, macros, RTA-driven |
| src/feedbacks.js | ~150 | Boolean feedbacks: mute state, enabled states, meter thresholds |
| src/variables.js | ~200 | Variable definitions + initial defaults: 200+ variables including meters |
| src/presets.js | ~500 | Stream Deck button templates: 200+ presets in 24 categories |
| src/upgrades.js | 3 | Version migration scripts (empty) |

## Connection Lifecycle

### Control Port (19272)

init() → _initTcp()

│

├─ TCPHelper auto-connects

├─ 'connect' → clear buffer, start handshake timeout (10s)

│

├─ WAIT_HELLO ← "HiQnet Console"

│   └─ send: connect administrator <password>

│

├─ AUTHENTICATING ← "connect logged in as administrator"

│   └─ send: ls "\\Preset"

│

├─ DISCOVERING ← module names until "endls"

│   └─ _finalizeTopology() → set topology variables

│   └─ updateActions/Feedbacks/Variables (topology-aware)

│   └─ _readAllState() → ~176 get commands at 5ms spacing

│   └─ _initDspMeters() → start port 19274 connection

│

└─ READY ← parse responses, update state, push variables

└─ _subscribeAll() → sub commands for real-time push

└─ _startKeepalive() → get Class_Name every 30s

### DSP Meter Port (19274)

_initDspMeters()

│

├─ Raw TCP socket (not TCPHelper — no auto-reconnect needed)

├─ "Started Dspcmd processor" → ready

│

├─ _discoverMeterIds()

│   └─ sends: module "OA/<module>" meterids (7 modules)

│   └─ collects hex IDs for RTA, input, output, comp, limiter meters

│

└─ _startMeterPoll() → setInterval at 200ms

└─ _pollMeters()

├─ sends: 31 RTA reads + 8 level reads = 39 commands per cycle

├─ _meterReadCallback maps responses to pa2State.meters

└─ _computeMeterVisuals() → unicode bars, peak tracker, formatted values

## State Management

### pa2State Object

{

device: { model, name, version },

topology: { modules, stereoGeq, leftGeq, rightGeq, hasHigh, hasMid, hasLow, ... },

geq: { enabled, mode, bands: { 1: gain, ..., 31: gain } },

peq: { High/Mid/Low: { enabled, filters: { 1-8: { Type, Frequency, Gain, Q, Slope } } } },

autoeq: { enabled, mode, filters },

afs: { AFS: bool, FilterMode, ContentMode, MaxFixedFilters, LiftTime },

compressor: { compressor: bool, threshold, gain, ratio, overeasy },

limiters: { High/Mid/Low: { limiter: bool, threshold, overeasy } },

mutes: { HighLeft, HighRight, MidLeft, MidRight, LowLeft, LowRight },

subharmonic: { enabled, master, lows, highs },

generator: { mode, level },

inputDelay: { enabled, ms },

outputDelays: { High/Mid/Low: { enabled, ms } },

rta: { rate, offset },

crossover: { Band_1/Band_2/Band_3/MonoSub: { hpType, lpType, hpFreq, lpFreq, gain, polarity } },

preset: { current, changed },

meters: { rta: [31 floats], inputL, inputR, compInput, compGR, limInput, limGR, outputHL, outputHR },

autoNotch: { active, slotsUsed, pending, lastDetectFreq, lastDetectAction },

}

### Data Flow

PA2 response line

│

├─ parseResponse(line) → { module, param, value, ... }

│   (pa2-protocol.js — pure function, no side effects)

│

├─ _updateStateFromParsed(result)

│   ├─ updates pa2State

│   ├─ computes formatted variables (*_fmt)

│   ├─ pushes to Companion: setVariableValues(vars)

│   └─ triggers: checkFeedbacks()

│

└─ Stream Deck buttons update via variable substitution

## Command Sending

Two modes:

### Spaced (5ms) — `sendCommands(cmds)`

For general operations where order matters. Commands sent sequentially with 5ms delay.

### Burst (0ms) — `sendCommandsBurst(cmds)`

For GEQ updates and time-critical operations. All commands joined with \n and sent as a single TCP write. PA2 accepts burst writes without dropping commands (tested: 62 commands in 1ms, zero drops).

Used by: geq_increment, geq_decrement, macro_auto_eq_from_rta, POST /geq, POST /eq/auto, POST /eq/curve.

## Subscription System

After initial state load, the module subscribes to critical parameters via the sub command. The PA2 pushes subr notifications when subscribed values change — no polling needed for these:

- All 6 output mutes

- Preset current/changed

- AFS enable/mode

- Compressor enable/threshold

- Generator mode/level

- GEQ enable/mode

- Room EQ enable

- Subharmonic/limiter/PEQ enable

- Input delay enable/amount

- Wizard state/event (detects front-panel AutoEQ runs)

## Wizard State Monitoring

When a PA2 wizard becomes active (front-panel AutoEQ, for example):

- subr notification: WizardState changes from "Inactive" to "Active"

- Module logs a warning

- When wizard finishes (state returns to "Inactive"), module calls _readAllState() to re-sync all parameters that may have changed

## HTTP Bridge

handleHttpRequest() in main.js routes requests by path:

| Path | Method | Handler |
| --- | --- | --- |
| /app | GET | Self-hosted HTML control page |
| /ping | GET | Connection status |
| /state | GET | Full pa2State |
| /topology | GET | Topology object |
| /rta | GET | 31-band RTA spectrum |
| /geq | GET | Current GEQ state |
| /geq | POST | Set GEQ bands (burst) |
| /meters | GET | All live meter values |
| /eq/auto | POST | Auto-EQ from RTA |
| /eq/curve | POST | Apply target curve |
| /loop | GET | Everything in one call |
| /command | POST | Generic action dispatch |
| /detect | POST | DoneWellAudio feedback detection |
| /recommendations | GET | Pending notch suggestions |
| /approve | POST | Approve/reject notches |
| /notches | DELETE | Clear auto-placed notches |

All responses include CORS headers. API key checked if configured.