# dbx DriveRack PA2

Control your dbx DriveRack PA2 loudspeaker management system via Ethernet. Full control of every parameter plus live RTA spectrum, input/output meters, and compressor/limiter gain reduction — features no other PA2 controller provides.

## Setup

- Connect PA2 to your network via Ethernet cable

- Find the PA2's IP address: Utility > System Info on the front panel

- In Companion, add the dbx DriveRack PA2 module

- Enter the IP address, port (default 19272), and password (default: administrator)

- Module auto-connects, discovers topology, reads all parameters, and starts meter polling

## Features

### Full Parameter Control

Every PA2 parameter is available as actions, feedbacks, and variables:

- 31-Band GEQ — individual bands, quick curves, flat, enable/disable

- 8-Band PEQ per output — filter type, frequency, gain, Q, slope, flatten/restore

- Room EQ / AutoEQ — Flat, Manual, AutoEQ modes

- AFS — Live/Fixed mode, content type, filter count, lift time, clear

- Compressor — threshold, gain, ratio, OverEasy

- Limiters per output band

- Crossover — HP/LP filter types, frequencies, gain, polarity

- Output Mutes — all 6 channels with toggle

- Delays — input (0-100ms), output per band (0-10ms)

- Subharmonic Synth — master, lows, highs

- Signal Generator — Off/Pink/White, level

- RTA — rate, offset

- Preset Recall — switch between presets 1-75

### Live DSP Meters (Exclusive Feature)

Real-time metering via the PA2's DSP interface (port 19274):

- 31-Band RTA Spectrum — live frequency analysis from the PA2's RTA mic

- Input Levels — Left/Right in dB

- Output Levels — per output channel in dB

- Compressor Gain Reduction — see how hard the compressor is working

- Limiter Gain Reduction — see limiter activity

- Visual bar graphs using unicode characters on Stream Deck buttons

### Smart Macros

One-press workflow buttons:

- Show Open — unmute all, AFS on, compressor on, generator off

- Show Close — mute all, generator off, clear AFS live filters

- Soundcheck — unmute, enable processing, flatten GEQ

- Ring Out — AFS Fixed mode, 12 filters, clear all

- Panic Mute — instant silence (mutes + generator off)

- Safe Unmute — unmute only if generator is off

- Speech Mode / Music Mode / DJ Mode / Intermission — complete scene changes

- Band Changeover — mute, flatten everything, reset for next act

- Monitor Check — safe pink noise with limiter protection

- Vocal Focus / De-Mud / De-Ess / Low Cut / Loudness — EQ sculpting

- Full Reset — nuclear option, everything to safe defaults

### RTA-Driven Actions

- Auto-EQ from RTA — reads live spectrum, computes inverse GEQ corrections to flatten the room

- Cut Peak — finds the loudest RTA band and cuts it

- Boost Weak — finds the quietest band and boosts it

- RTA Snapshot — save current spectrum for A/B comparison

- RTA Compare — show dB difference between live and snapshot

### GEQ Fader Mode

- Increment/Decrement — +dB / -dB per press with configurable step size

- Burst mode — changes apply in under 1ms

## Stream Deck XL Layout Suggestions

### Page 1: Show Control (4x8)

Row 1: HIGH L, HIGH R, MID L, MID R, LOW L, LOW R, MUTE ALL, SAFE UNMUTE

Row 2: SHOW OPEN, SHOW CLOSE, SOUNDCHECK, RING OUT, PANIC MUTE, GEN OFF, PRESET, STATUS

Row 3: AFS ON, AFS MODE, CLR LIVE, CLR ALL, COMP, LIMITER, PEQ, ROOM EQ

Row 4: Input meter, Output meter, Comp GR, Lim GR, Signal, RTA Peak, [blank], [blank]

### Page 2: GEQ + RTA

Row 1: RTA Lows, RTA Lo-Mid, RTA Hi-Mid, RTA Highs, PEAK, AUTO EQ, CUT PEAK, GEQ FLAT

Row 2-4: GEQ band increment/decrement buttons for key frequencies

### Page 3: Smart Macros

Speech, Music, DJ, Intermission, Changeover, Monitor Check, Full Reset, plus EQ sculpting

## HTTP Bridge

External web apps (like donewellaudio.com) can control the PA2 through Companion's HTTP API.

Setup:

- Enable HTTP API in Companion Settings > Network

- Note your Companion port (default 8000) and instance label

- URL format: http://<companion-ip>:<port>/instance/<label>/<endpoint>

Key Endpoints:

- GET /ping — connection status

- GET /loop — everything: RTA + GEQ + meters + mutes (for polling)

- GET /rta — live 31-band spectrum

- POST /geq — set GEQ bands (burst mode)

- POST /eq/auto — auto-EQ from current RTA

- GET /app — self-hosted control page (no mixed content issues)

Optional: Set an API key in module config to require X-Api-Key header.

See docs/HTTP-API.md for the full reference.

## Troubleshooting

- Can't connect: PA2 needs an IP via DHCP or static. Check Utility > System Info.

- Auth failed: Default password is administrator. Check PA2 Utility > System Settings.

- GEQ commands affect both channels: Normal for dual-mono topology. Module auto-detects and sends to both Left and Right GEQ.

- Missing Mid/Low actions: Normal for full-range (High-only) configurations. Module adapts to your topology.

- No RTA data: Ensure the RTA mic is connected to the PA2's front XLR input.

- HTTP bridge not responding: Enable HTTP API in Companion Settings > Network.

- Meters not updating: The DSP meter connection (port 19274) starts automatically after the main connection authenticates. Check Companion logs for "DSP meter port connected".

- Handshake timeout: If the PA2 doesn't respond within 10 seconds, the module disconnects and retries. Check network connectivity.