PA2 AUTO-NOTCH PIPELINE

Detection to Cut — Merged Dual-Audit

DoneWell Audio | March 2026 | 31 findings across 5 phases

# 1. Executive Summary

Merged code audit of the PA2 Auto-Notch Pipeline from two independent reviews. Every line of pipeline code was read. Covers the full path from browser feedback detection through HTTP bridge to PA2 hardware PEQ/GEQ writes.

| Phase | Count | Key Issues |
| --- | --- | --- |
| Phase 0: Safety Gates | 4 | Detect runs before state loaded, No command queue, No request validation |
| Phase 1: Critical | 4 | Both Mode Double-Cuts, Hybrid Drops Confidence/Q, No Transactional Ack |
| Phase 2: High | 10 | Approve Global Slot Check, Approve Wrong Output, Routing Fails HPF Bypass |
| Phase 3: Medium | 7 | Depth Ignores Q, Bell Not Notch, AbortSignal Leak |
| Phase 4: Architecture | 6 | Wire closedLoopEQ, Auto-Release Notches, Wire Panic Mute |

# 2. Architecture

| # | File | Lines | Role |
| --- | --- | --- | --- |
| 1 | lib/pa2/advisoryBridge.ts | 234 | Advisory to GEQ/PEQ mapping |
| 2 | hooks/usePA2Bridge.ts | 453 | Polling, auto-send, state |
| 3 | lib/pa2/pa2Client.ts | 226 | HTTP client |
| 4 | types/pa2.ts | 276 | TypeScript interfaces |
| 5 | companion src/main.js | 1739 | HTTP handler, TCP, notch tracking |
| 6 | companion src/pa2-protocol.js | 965 | Command builder, parser |
| 7 | lib/pa2/closedLoopEQ.ts | 129 | Verify + deepen cycle (unwired) |
| 8 | lib/pa2/modeSync.ts | 101 | Mode-aware PA2 config sync |

# 3. Workflow Timeline

Detection-to-cut flow with timing:

| Phase | Timing | What Happens |
| --- | --- | --- |
| Topology | t=0 | Module reaches READY. stateLoaded still false. |
| State read | t=0..880ms | ~176 get commands at 5ms spacing |
| State loaded | t=5s min | Timer-driven, not acknowledgement-driven |
| POST /detect | any time | Accepted if READY, even before stateLoaded |
| Route+dedup | immediate | Depends on in-memory crossover and slot ledger |
| PEQ writes | immediate | sendCommands() 5ms spacing, not awaited |
| HTTP success | immediate | Returned before PA2 acknowledges write |

# 4. Severity Reference

| Severity | Meaning | GEQ Depth |
| --- | --- | --- |
| RUNAWAY | Exponential growth | -12 dB |
| GROWING | Sustained rise | -6 dB |
| RESONANCE | Room mode | -3 dB |
| WHISTLE | Pure tone | -4 dB |

# 5. Phase 0: Safety Gates

## P0-1: Detect runs before state loaded

CRITICAL   main.js:1373 vs 473

Problem

/detect checks READY but not stateLoaded. Crossover zeroed, routes all to High.

Fix

Gate on stateLoaded. Prefer ack-driven completion.

## P0-2: No command queue

CRITICAL   main.js:1063-1076

Problem

sendCommands not awaited. Concurrent requests interleave PEQ writes.

Fix

Serialized command queue.

## P0-3: No request validation

HIGH   main.js:1378-1399

Problem

det.hz/confidence can be undefined, bypasses threshold.

Fix

Validate hz, confidence, q, type.

## P0-4: Config || vs ??

HIGH   main.js:1380

Problem

threshold || 0.8 treats configured 0 as falsy.

Fix

Change || to ??.

# 6. Phase 1: Critical

## C-1: Both Mode Double-Cuts

CRITICAL   usePA2Bridge.ts:331-371

Problem

Same advisory as GEQ AND PEQ. -6dB becomes -12dB.

Fix

Partition: narrow to PEQ, broad to GEQ.

## C-2: Hybrid Drops Confidence/Q

CRITICAL   usePA2Bridge.ts:312-317

Problem

Hardcodes confidence:0.9, drops Q.

Fix

Forward actual values.

## C-3: No Transactional Ack

CRITICAL   main.js:1451

Problem

Slots mutated before PA2 confirms. Partial writes drift ledger.

Fix

Pending state, finalize on echo.

## C-4: Slot Ownership Memory-Only

CRITICAL   main.js:1728-1734

Problem

Lost on reconnect. Never rebuilt from PEQ state.

Fix

Persist and reconcile.

# 7. Phase 2: High

## H-1: Approve Global Slot Check

HIGH   main.js:1494-1500

Problem

Checks ALL slots not per-output.

Fix

Filter by output.

## H-2: Approve Wrong Output

HIGH   main.js:1513

Problem

Response hardcodes High.

Fix

Use approveOutput.

## H-3: Routing Fails HPF Bypass

HIGH   main.js:856-869

Problem

hpFreq=0 is falsy.

Fix

Check lpFreq.

## H-4: GEQ Accumulates Per Cycle

HIGH   advisoryBridge.ts:119-131

Problem

Adds correction every 1s.

Fix

Track applied, send deltas.

## H-5: Macro Param Keys

HIGH   pa2-protocol.js:729-742

Problem

{level:} not {value:}.

Fix

Fix keys.

## H-6: Pending Keyed by Freq

HIGH   main.js:1488

Problem

Not by detection ID.

Fix

Server-side detectionId.

## H-7: Approve No Dedup

HIGH   main.js:1493-1513

Problem

Can stack near-duplicate cuts.

Fix

Share dedup logic.

## H-8: Release Only Zeroes Gain

HIGH   main.js:1541-1548

Problem

No restore of prior state.

Fix

Save/restore slot state.

## H-9: PEQ Mid/Low Not Subscribed

HIGH   main.js _subscribeAll

Problem

Only High gets live push.

Fix

Subscribe all outputs.

## H-10: closedLoopEQ Unwired

HIGH   closedLoopEQ.ts:90-128

Problem

Exists but never called.

Fix

Wire into auto-send.

# 8. Phase 3: Medium

## M-1: Depth Ignores Q

MEDIUM   advisoryBridge.ts:58-68

Problem

Q=16 same depth as Q=4.

Fix

Q-scaling.

## M-2: Bell Not Notch

MEDIUM   main.js:1446

Problem

Wider skirt.

Fix

Use Notch type.

## M-3: AbortSignal Leak

MEDIUM   pa2Client.ts:215-225

Problem

144K listeners over 4h.

Fix

AbortSignal.any().

## M-4: PEQ Enable Not Checked

MEDIUM   main.js:1446

Problem

Notch into disabled PEQ.

Fix

Check/enable first.

## M-5: Dead Telemetry

MEDIUM   variables.js:169-194

Problem

detect_active never updated.

Fix

Wire or remove.

## M-6: Poll/Send Race

MEDIUM   usePA2Bridge.ts

Problem

Stale GEQ data.

Fix

Freshness check.

## M-7: modeSync Boolean String

MEDIUM   modeSync.ts:93

Problem

Sends string false which is truthy.

Fix

Send boolean.

# 9. Phase 4: Architecture

## A-1: Wire closedLoopEQ

LOW   closedLoopEQ.ts

Problem

Verify-deepen cycle exists unwired.

Fix

Call from auto-send.

## A-2: Auto-Release Notches

LOW   usePA2Bridge.ts

Problem

No release on resolve.

Fix

Watch resolved, call release.

## A-3: Wire Panic Mute

LOW   usePA2Bridge.ts

Problem

panicMuteEnabled unwired.

Fix

Check on RUNAWAY.

## A-4: Slot Prioritization

LOW   main.js /detect

Problem

Full=skip.

Fix

Replace least important.

## A-5: AFS Coordination

LOW   main.js /detect

Problem

No AFS proximity check.

Fix

Query AFS before notch.

## A-6: No HTTP Handler Tests

LOW   test/

Problem

Only protocol tested.

Fix

Add handler + notch tests.

# 10. Engineering Guide

## Beginner

Audio feedback = speaker sound re-enters mic, amplifies one frequency. This pipeline detects problem frequencies and tells PA2 hardware to cut them.

- GEQ: 31 fixed-width faders for broad corrections

- PEQ: Variable-width filters for surgical notches

- Q Factor: narrowness. Q=16 very narrow, Q=2 very wide

- Severity: RUNAWAY=emergency, GROWING=worsening, RESONANCE=mild

## Intermediate

Stage 1 (Browser): FFT 50x/sec, 7 algorithms fuse scores. Stage 2 (Bridge): Map freq to band, severity to dB. Stage 3 (Companion): HTTP to TCP, PEQ slots, output routing.

## Advanced

Confidence (3 layers)

| Layer | Default | Purpose |
| --- | --- | --- |
| Advisory creation | Varies by algo | Worth reporting? |
| Bridge auto-send | 0.7 (PEQ) / 0.6 (GEQ) | Worth sending? |
| Companion /detect | 0.8 (configurable) | Worth cutting? |

Dedup (4 layers)

- Advisory: 500ms global + 300ms per-band cooldown

- Bridge: deepest cut wins per GEQ band

- Companion: 1/3-octave proximity (ratio <= 1.26)

- Auto-send: 1000ms minimum interval

HiQnet Protocol

set "\\Preset\High Outputs PEQ\SV\Band_1_Frequency" 2500.00

Text-based TCP on port 19272. SET uses raw values, GET responses include unit suffixes (dB, kHz, %).

Connection State Machine

IDLE > WAIT_HELLO > AUTHENTICATING > DISCOVERING > READY

READY fires before stateLoaded. Timer-based: max(cmds*25ms, 5000ms). Not acknowledgement-driven.

# 11. Implementation Roadmap

| Phase | Items | Key Files | Status |
| --- | --- | --- | --- |
| Phase 0: Safety Gates | 4 | main.js | Not started |
| Phase 1: Critical | 4 | usePA2Bridge.ts, main.js | Not started |
| Phase 2: High | 10 | main.js | Not started |
| Phase 3: Medium | 7 | advisoryBridge.ts, main.js, pa2Client.ts | Not started |
| Phase 4: Architecture | 6 | closedLoopEQ.ts, usePA2Bridge.ts | Not started |

End of Document