PA2 AUTO-NOTCH PIPELINE

Detection to Cut - Deep Code Audit

DoneWell Audio | March 2026 | 6 files | 3,893 lines | 11 bugs | 11 gaps | 4 optimizations

# 1. Executive Summary

Comprehensive code audit of the PA2 Auto-Notch Pipeline, covering the full signal path from browser-based feedback detection through to physical EQ cuts on dbx DriveRack PA2 hardware.

| Category | Count | Impact |
| --- | --- | --- |
| Critical Bugs | 5 | Double-cut audio, wrong output routing, data loss |
| High Bugs | 2 | GEQ accumulation, macros generate undefined |
| Medium Bugs | 4 | Wrong filter type, leak, race, Q-blind |
| Gaps | 11 | No verify, no release, no tests |
| Optimizations | 4 | Polling, signals, dedup, batching |

# 2. Architecture

| # | File | Lines | Role |
| --- | --- | --- | --- |
| 1 | lib/pa2/advisoryBridge.ts | 234 | Advisory to GEQ/PEQ mapping |
| 2 | hooks/usePA2Bridge.ts | 453 | Polling, auto-send, state |
| 3 | lib/pa2/pa2Client.ts | 226 | HTTP client |
| 4 | types/pa2.ts | 276 | TypeScript interfaces |
| 5 | companion src/main.js | 1739 | HTTP handler, TCP, notch tracking |
| 6 | companion src/pa2-protocol.js | 965 | Command builder, parser |

# 3. Workflow

Severity levels from feedback detection:

| Severity | Meaning | Depth |
| --- | --- | --- |
| RUNAWAY | Exponential growth | -12 dB |
| GROWING | Sustained rise | -6 dB |
| RESONANCE | Room mode | -3 dB |
| WHISTLE | Pure tone | -4 dB |

## Bridge Functions

- advisoriesToGEQCorrections() - log-space to nearest ISO band

- advisoriesToDetectPayload() - exact Hz, Q, confidence

- advisoriesToHybridActions() - decides GEQ vs PEQ

## Companion Processing

- Connection check (READY)

- Confidence filter (0.8)

- _routeToOutput() crossover routing

- 1/3-octave dedup

- PEQ slot allocation

- buildCommand(peq_filter)

# 4. Bugs

## B9: Both Mode Double-Cuts

CRITICAL   usePA2Bridge.ts:331-371

Problem

Same advisory sent as BOTH GEQ and PEQ. -6dB becomes -12dB.

Fix

Partition: narrow/urgent to PEQ, broad to GEQ.

## B1: Hybrid Drops Confidence/Q

CRITICAL   usePA2Bridge.ts:312-317

Problem

Hardcodes confidence:0.9, drops Q.

Fix

Forward actual confidence and Q.

## B4: /approve Global Slot Check

CRITICAL   main.js:1494-1500

Problem

Checks ALL slots not per-output.

Fix

Filter by output.

## B10: Routing Fails on HPF Bypass

CRITICAL   main.js:856-869

Problem

hpFreq=0 is falsy. All freqs route to High.

Fix

Check lpFreq instead.

## B3: /approve Wrong Output

CRITICAL   main.js:1513

Problem

Response says High even when cmd goes to Mid/Low.

Fix

Use approveOutput.

## B7: GEQ Accumulates Per Cycle

HIGH   advisoryBridge.ts:119-131

Problem

Adds correction every 1s. 3 cycles = 3x cut.

Fix

Track applied, send deltas.

## B11: Macro Param Keys Wrong

HIGH   pa2-protocol.js:729-742

Problem

{level:} but handler expects {value:}.

Fix

Fix keys.

## B6: Depth Ignores Q

MEDIUM   advisoryBridge.ts:58-68

Problem

Q=16 and Q=4 get same depth.

Fix

Q-scaling factor.

## B5: AbortSignal Leak

MEDIUM   pa2Client.ts:215-225

Problem

144K leaked listeners over 4h show.

Fix

AbortSignal.any().

## G8: Bell Not Notch

MEDIUM   main.js:1446

Problem

Bell has wider skirt.

Fix

Use Notch type.

# 5. Gaps

## G1: No Verification Loop

After notch, never checks if feedback stopped.

## G2: No Notch Release

Resolved advisories leave notches permanently.

## G3: Hardcoded 8 Slots

PA2 has 8 per output. 3-way=24 total.

## G4: No AFS Coordination

AFS may already cover the frequency.

## G5: No Depth Ramping

First detection=max depth.

## G6: Panic Mute Unwired

Setting exists but never checked.

## G11: No Tests

No tests for bridge layer.

# 6. Recommendations

| # | Bug | Fix | Effort |
| --- | --- | --- | --- |
| 1 | B9 | Partition both mode | 2h |
| 2 | B1 | Forward confidence/Q | 1h |
| 3 | B4+B3 | Per-output slots + response | 30m |
| 4 | B10 | Fix routing guard | 30m |
| 5 | B11 | Fix macro keys | 15m |
| 6 | B7 | Delta-only corrections | 2h |

# 7. Risk Matrix

|  | Low | Med | High |
| --- | --- | --- | --- |
| Certain | B8 | B7 | B9 |
| Likely | O1 | B1, B11 | B10 |
| Possible |  | B5, G10 | G1 |

# 8. Engineering Guide

## Beginner

Audio feedback = speaker sound re-enters mic. This pipeline detects and cuts it.

- GEQ: 31 fixed faders for broad corrections

- PEQ: Variable-width filters for surgical notches

- Q Factor: narrowness. Q=16 very narrow, Q=2 very wide

- Severity: RUNAWAY=emergency, GROWING=worsening, RESONANCE=mild

## Intermediate

Stage 1: FFT 50x/sec, 7 algorithms. Stage 2: Map freq to band, severity to dB. Stage 3: HTTP to TCP.

## Advanced

Confidence (3 layers)

| Layer | Default | Purpose |
| --- | --- | --- |
| Advisory | Varies | Worth reporting? |
| Bridge | 0.7/0.6 | Worth sending? |
| Companion | 0.8 | Worth cutting? |

Dedup (4 layers)

- Advisory: 500ms global + 300ms per-band

- Bridge: deepest cut wins

- Companion: 1/3-octave proximity

- Auto-send: 1000ms interval

End of Document